# Smart EduBuddy

System Workflow & Circuit Diagram

Complete Technical Documentation

Contents:

1. System Workflow: How It Works

2. Circuit Diagram (NodeMCU / ESP32)

3. Power Supply

4. Software Architecture

5. MQTT Communication Protocol

6. Testing & Troubleshooting

7. Safety Guidelines

# 1. System Workflow: How It Works

## Step 1: Teacher Sets Question

- Teacher opens "Teacher Panel" on their laptop.

- Teacher types a question (e.g., "What color is the sky?")

- Teacher selects the Correct Answer from the dropdown (e.g., "Color Blue").

- Teacher clicks "Send Question".

- Result:

- The question appears on the Dashboard Screen (Projector/TV).

- The system silently stores the "Correct Answer ID" (e.g., 02BLUE).

## Step 2: Student Responds

- Student looks at the question on the big screen.

- Student finds the physical card that matches their answer (e.g., The Blue Card).

- Student taps the card on the Smart EduBuddy Device (RFID Reader).

## Step 3: Device Processing

- RFID Reader detects the card.

- NodeMCU/ESP32 reads the unique ID from the card (e.g., UID: E3 4A B2).

- Code converts that detected ID into a readable Answer ID (e.g., 02BLUE).

- Device sends this message: "02BLUE" to the MQTT Cloud.

## Step 4: System Verification

- Web Dashboard receives the message "02BLUE".

- It checks: Is 02BLUE == Correct Answer ID?

- YES (Correct):

- Screen flashes GREEN.

- Audio plays: "Correct! Great Job!"

- +1 Score is added to the "Correct" scoreboard.

- NO (Wrong):

- Screen flashes RED.

- Audio plays: "Oops! Try Again!"

- +1 Score is added to the "Wrong" scoreboard.

# 2. Circuit Diagram (NodeMCU / ESP32)

Figure 1: Complete Circuit Schematic Diagram

## Components Needed:

- Microcontroller: NodeMCU (ESP8266) OR ESP32 Dev Module

- RFID Reader: MFRC522 (RC522)

- Buttons (Optional): 2x Push Buttons (for testing without cards)

- LED (Optional): Built-in LED is used for status.

## Wiring Connections:

### A. RFID Reader (RC522) to Microcontroller

| RC522 Pin | NodeMCU (ESP8266) | ESP32 |
| --- | --- | --- |
| SDA (SS) | D4 (GPIO 2) | GPIO 5 |
| SCK | D5 (GPIO 14) | GPIO 18 |
| MOSI | D7 (GPIO 13) | GPIO 23 |
| MISO | D6 (GPIO 12) | GPIO 19 |
| IRQ | Not Connected | Not Connected |
| GND | GND | GND |
| RST | D3 (GPIO 0) | GPIO 22 |
| 3.3V | 3.3V (3V3) | 3.3V (3V3) |

⚠️ WARNING: Do NOT connect RC522 to 5V (Vin). It will damage the reader! Always use 3.3V.

# 3. Power Supply

Connect the NodeMCU / ESP32 to your laptop or a USB charger using a Micro-USB cable.

## Power Requirements:

- Input Voltage: 5V DC (via USB)

- Current Draw: ~200mA (typical), ~500mA (max)

- Power Consumption: ~1W average

- Power Source Options: USB wall adapter, laptop USB port, power bank

## Recommended Power Sources:

- Phone charger (5V, 1A minimum)

- Laptop USB port (for testing)

- Power bank (for portable operation)

# 4. Software Architecture

## System Components:

### NodeMCU Firmware

- Language: Arduino C++

- Libraries: MFRC522, PubSubClient, ESP8266WiFi

- Functions: WiFi connection, RFID reading, MQTT publishing

### Web Dashboard

- Technology: HTML5, CSS3, JavaScript

- MQTT Library: MQTT.js (WebSocket)

- Functions: Display questions, show feedback, track scores

### Teacher Control Panel

- Interface: Web-based (responsive design)

- Features: Question creation, preset options, game controls

# 5. MQTT Communication Protocol

The system uses MQTT (Message Queuing Telemetry Transport) for real-time communication between all components.

## MQTT Topics:

| Topic | Purpose |
| --- | --- |
| edubuddy/question | Teacher sends questions to dashboard |
| edubuddy/answer | NodeMCU sends student answers |
| edubuddy/status | Device online/offline notifications |
| edubuddy/control | System control commands (reset, clear) |

## Message Format Example:

Question Message: QUESTION|02BLUE|Find the Color Blue

Answer Message: A1B2C3D4

# 6. Testing & Troubleshooting

## Initial Testing Steps:

- Power On Test: Check if NodeMCU power LED lights up

- WiFi Connection: Verify connection via Serial Monitor

- RFID Test: Place card on reader, check Serial Monitor for UID

- MQTT Test: Verify "Connected" status on dashboard

## Common Issues & Solutions:

### Issue 1: RFID Not Reading Cards

- Verify 3.3V power connection (NOT 5V!)

- Check all SPI wire connections

- Try different RFID cards

### Issue 2: WiFi Won't Connect

- Double-check WiFi SSID and password

- Ensure using 2.4GHz network (ESP8266 doesn't support 5GHz)

- Move NodeMCU closer to router

### Issue 3: Dashboard Shows Offline

- Refresh browser page

- Check internet connection

- Verify MQTT broker is accessible

# 7. Safety Guidelines

## Electrical Safety:

- Always use 5V USB power only

- Never connect RFID reader to 5V - use 3.3V only!

- Avoid touching components while powered on

- Use insulated enclosure for classroom use

## Physical Safety:

- No exposed sharp components

- Secure all wire connections

- Keep device away from water and liquids

- Supervise children during use

## Data Safety:

- No personal student data is stored

- System only tracks anonymous scores

- All communication is on local network or encrypted MQTT

Smart EduBuddy - IoT Educational System

Version 1.0 | Complete Technical Documentation

For more information, see the complete documentation package